Name ___________________________________________

Class ___________________________________________

- The airspeed indicator is showing 0 airspeed even though the plane is in motion. All the other instruments on the aircraft appear to be functioning normally. The pilot suspects a pitot-static malfunction. What might be the problem?

Ram air port blockage

Drain hole blockage

Static port blockage

Both ram port and drain hole blockage

- A pilot’s VSI shows the following. What is the vertical speed of the aircraft?

Editorial credit: AOPA

- 0.6 ft/min climb

600 ft/min climb

0.6 ft/min descent

600 ft/min descent

- How does a gyroscope respond to a deflective force when it is powered?

- It moves in the direction of deflection and the deflection occurs 90 degrees in the direction of rotation from the point at which the force is applied.

It moves in the direction of deflection and the deflection occurs 90 degrees opposite the direction of rotation from the point at which the force is applied.

It moves opposite the direction of deflection and the deflection occurs 90 degrees in the direction of rotation from the point at which the force is applied.

It moves opposite the direction of deflection and the deflection occurs 90 degrees opposite the direction of rotation from the point at which the force is applied.

- What does the white arc on the airspeed indicator indicate?

- The safe operating range for the flaps

The maximum speed with flaps extended

The normal operating range of the aircraft

The power-off stall speed at maximum takeoff weight

- What information is processed by the air data computer (ADC)?

- Information from gyroscopic instruments

Information about direction and heading

Information about static and ram pressure

None; it is used to store information collected during flight.

- How is the groundspeed of an aircraft calculated?

- Headwinds are added and tailwinds are subtracted from the true air speed

Tailwinds are added and headwinds are subtracted from the true air speed

Headwinds are added and tailwinds are subtracted from the calibrated air speed

Tailwinds are added and headwinds are subtracted from the calibrated air speed

- What information is provided on a compass correction card?

- The amount of adjustment required to correct for northerly turning error

The amount of adjustment required to correct for magnetic deviation

The amount of adjustment required to correct for magnetic variation

The amount of adjustment required to correct for acceleration error

- How should a pilot adjust their altimeter before takeoff if operating at an airport where no altimeter setting is provided?

- The altimeter should be adjusted until it displays 0.

The altimeter should be adjusted until it displays the airport’s elevation.

The altimeter should be adjusted until it displays the elevation at sea level.

The altimeter should be adjusted until it displays the lowest possible elevation.

- An aircraft’s static port is blocked but the ram air port and the drain hole remain unobstructed. What instruments would become unreliable? Select all that apply.

- Airspeed indicator

Altimeter

Attitude indicator

Heading indicator

Turn coordinator

Vertical speed indicator

- A pilot is turning the plane north. Which of these guidelines should they follow?

- The pilot should stop the turn after the desired heading because the compass leads the turn

The pilot should stop the turn before the desired heading because the compass leads the turn

The pilot should stop the turn after the desired heading because the compass lags the turn

The pilot should stop the turn before the desired heading because the compass lags the turn

- What do the white horizontal lines on the attitude indicator indicate?

Editorial credit: Leland Roys

- The degrees of bank

The degrees of pitch

The height above the ground

The rate of turn

- What is denoted by VLE?

- The speed at which an aircraft will gain the greatest amount of altitude over a given distance

The speed at which a pilot can safely fully deflect one control surface in smooth air

The maximum speed for flying with landing gear extended

The never exceed speed

- The pilot needs to regularly reset the compass card of the heading indicator to match the magnetic compass. Which of these will cause the compass card to drift so that it needs to be reset?

- A change in aircraft altitude

A change in aircraft location

The rotation of the Earth

The magnetic field of the Earth

- How should a pilot adjust the reading from their magnetic compass to calculate true north near Richmond, Virginia?

Editorial credit: SkyVector

- Add 10°

Subtract 10°

018°

001°

- What type of speed is read directly from the airspeed indicator?

- Calibrated airspeed

Groundspeed

Indicated airspeed

True airspeed

- Acceleration in which of these situations would cause the magnetic compass to deflect toward the north?

- Accelerating when flying eastward in the Northern Hemisphere

Decelerating when flying westward in the Northern Hemisphere

Accelerating when flying northward in the Northern Hemisphere

Decelerating when flying southward in the Northern Hemisphere

- What is the definition of density altitude?

- The indicated altitude when the pressure is set to 29.92 inHg

The altitude above mean sea level

The pressure altitude corrected for non-standard temperature

The altitude above the ground

- What instruments are controlled by the attitude heading and reference system (AHRS) in aircraft with an electronic flight display (EFD)? Select all that apply.

- Airspeed indicator

Attitude indicator

Heading indicator

Slip/Skid indicator

Turn indicator

Vertical speed indicator

- Which of these instruments do not rely on a gyroscope? Select all that apply.

- Altimeter

Attitude indicator

Heading indicator

Inclinometer

Turn and slip indicator

- How are predicted future speed, altitude, and heading displayed on EFDs?

- As green arrows

As green lines

As magenta arrows

As magenta lines

- What is one indication that there is a gyroscopic instrument failure on an aircraft? How should the pilot respond if this happens when flying in instrument conditions (i.e., without outside visual references)?

- What are two benefits of EFDs over analog gauges? Explain how each benefit contributes to flight safety.

- Explain how static, dynamic, and total pressure are related.

- Explain why turning errors are greatest near the poles and least near the equator.

- During flight, a pilot notices that adding power and raising the aircraft’s nose does not result in changes in the altimeter. Describe two actions the pilot should take to react to this situation.