HHO a viable means For reducing emissions

Jeffery Morse

Brandman University

Methods and Applications Final

Author Note

For correspondence concerning this paper Jeffery Morse can be reached at

jmorse@mail.brandman.edu

Abstract

This paper explores current HHO technologies. It concludes with the findings that an HHO cell is a viable alternative for further exploration and experimentation. Further, it finds that the cost of electric production is high in the previous experiment due to the nature of HHO cells to have a high current draw at voltages at or around auto batteries and charging systems. Alternative voltages of 30 volts or higher are recommended for use. Further findings include alternating current to be acceptable only when put through a half or full wave rectifier to create a pulse. Constant d.c. voltages are shown to have too high a cost in heat production so a pulse is recommended. An HHO cell can be tuned to have very little or even 0 current draw when the frequency of the pulse is adjusted. In this experiment, it was found that a voltage of about 30 volts at 22440 Hz was sufficient to produce 28 mL of gas per minute with a very small HHO cell that cost less than 20 dollars to produce. Finally, this technology is recommended for further study, but is not recommended for continual used as better alternatives are available.

Keywords: HHO, Hydroxy, Hydroxyl, Browns Gas, Gas Cell, fuel from water, alternative fuels.

LIST OF ABBREVIATIONS AND NOMENCLATURE

HHO : Hydroxy, Hydroxyl, BG Browns Gas, SI : Spark Ignition, CI : Compression Ignition, LPG : Liquefied Petroleum Gas, CO : Carbon monoxide, HC : Hydrocarbon, CO2 : Carbon dioxide, NOx : Nitrogen oxide, HEV : Hybrid Electric Vehicle, H2ICE : Hydrogen fueled Internal Combustion Engine, KOH : Potassium Hydroxide, NaOH : Sodium Hydroxide, NaCl : Sodium Chloride, EEW: : Electrically Expanded Water W : Watt, SFC : Specific Fuel Consumption, HECU : Hydroxy Electronic Control Unit, TDC : Top Dead Center BDC : Bottom Dead Center, EEPROM : Electrically Erasable Programmable Read-Only Memory, F : Force, T : Torque, rev. : Revolution, Pb : Brake Power, bmep : Brake Mean Effective Pressure BSFC : Brake Specific Fuel Consumption, bth h : Thermal Efficiency, CR : Compression Ratio, AFR : Air-Fuel Ratio, EGR : Exhaust Gas Recirculation, rpm : Revolutions Per Minute, CATIA : Computer Aided Three-dimensional Interactive Application, V : Volt, A : Ampere, CCM : Cubic Centimeters per Minute, SLM : Standard Liters per Minute, DC : Direct Current, AC : Alternating Current mF : Micro Farad, MOSFET : Metal Oxide Semiconductor Field Effect Transistor, PWM : Pulse Width Modulation, f : Frequency, MAP : Manifold Absolute Pressure, ADC: Analog to Digital Converter, DAC: Digital to analog converter, PLC: Programmable Logic Controller, STP: Standard temperature and pressure: 298.15 K, 101.325 kPa

HHO a viable means or reducing emissions

Any business person will tell you that the problem with dependence on any non-renewable energy source such as fossil fuels is that it has no sustainability and thus in the long term it is a strategically bad plan. At best such efforts are a race, the dash for the cash until such resources run dry.  We in the U.S. have, never the less, chosen fossil fuels as our main source of energy.  We use it in a spectrum that ranges from the creating of electricity to traveling the world.  Unfortunately, too often we dismiss evidence linking climate change and air pollution to fossil fuel consumption.  These changes have been so gradual that many believe them not to be true.  Cars and the pollution from cars are the single greatest polluter in our nation’s cities (Tekin and Çavuşoğlu, 1997). It is hard to imagine that as little as 100 years ago electric power and the internal combustion engine (ICE) were rarely even heard of.  Auto pollution comes from by-products of the combustion of fossil fuel (exhaust) and from the vapor of the fuel itself. Since this combustion is not perfectly efficient an automobile emits several types of pollutants. These are: Hydrocarbons (HC), which are basically unburned fuel and Nitrogen oxides (NOx) which are created by the high-pressure combustion of nitrogen and oxygen.  Like hydrocarbons NOx interferes with the formation of ozone and contributes to acid rain. Carbon Monoxide CO is a product of the incomplete combustion of fossil fuels.  When inhaled CO reduces the flow of oxygen in the bloodstream. This is very dangerous, especially to persons with heart disease. Carbon Dioxide (CO2), according to the EPA, is a main contributor to global warming as a greenhouse gas (EPA, 1994). Gasoline and diesel fuels are mixtures of HC (Hydro Carbons), compounds which contain hydrogen and carbon atoms. Smog is proportional to the amount of unburned HCs present in exhaust. Many things in an engine can produce emissions; for example advanced timing or a bad catalytic converter may produce HC’s and though we cannot see CO -- a colorless and odorless killer-- these gases are common emissions and do not need any failure from a engine component to be produced.  NOx (Nitrogen Oxides) are also very harmful to humans and contribute to smog, greenhouse effects, acid rain, and form toxic compounds which are also toxic to humans. Because of this, maximum limits on NOx emissions are continually being reduced (Walsh, 2001)(Bowman, 1992). NOx, HC’s and CO top the list as the worst offenders for health and smog (Musmar, 2011). Catalytic converters introduced in 1975 significantly reduced hydrocarbon and carbon monoxide emissions; however, lead inactivates the catalyst in catalytic converters so oil companies had no choice but to once again remove lead from fuel. Today normal petrol combustion has the following emissions:  Carbon monoxide 1-2% of volume, HC <.25% of volume, Nitrogen 71% of volume, CO2 14% of volume, water vapor 12% of volume, NOx <.25% of volume, SOx < .03% of volume (Rajeshkumar et al, 2016). (Figure 1) shows the produced by the EPA (Environmental Protection Agency) shows the

Figure 1: EPA graph of emission producers.

contributors to emissions.

The EPA functions under The “Clean Air Act” of 1970 to regulate motor vehicles. This has led to devices to manage the exhaust caused by burning fossil fuels. The first regulation was through engine design, followed by the catalytic converter, the Oxygen Sensor and finally the Exhaust Gas Recirculation (EGR) to manage NOx (EPA, 1994). According to the EPA graph we see that the production of electricity is the biggest offender in total, not just in cities, which is followed by transportation and then industry. According to Harper (2007), electricity is produced mainly by fossil fuels, coal and natural gas being the most widely used.  On the other hand, gasoline and diesel are the most commonly used fossil fuels in transportation.  Reportedly fossil fuel accounts for approximately 90% of our energy consumption.  As a result of this fact fossil fuel-based energy sources are facing increasing pressure on a host of environmental fronts, including a serious challenge for coal burning facilities to reach the new greenhouse gas reduction targets, but we are not likely to do enough in time to reduce expected damages. As a result, we must come up with more ways to curb emission and greenhouse gas production from fossil fuel burning machinery (Herzog et al., 2006).  We have found a technology that Hydroxy (HHO) that we hypothesize would show promising emission reductions.

A closer look at the problems from emissions

Figure 4. Cardio Disease in China

Figure 3. China verses world CO2

Figure 2.  Chinese Coal Consumption

Recent changes in the world’s economy afford us an opportunity to see our end if we stay on our current course.  Global warming and emissions are a real world problems faced by the scientific community. As a result of the short time frame and exorbitant growth we can view China as a petri dish showing the end result of an economy based on fossil fuels (Figure 2). This provides us with the best evidence to date of the negative effects we can expect if we continue in fossil fuel dependency. (Figure 3) shows the increase in CO2 due to increased industry and auto use shown in (Figure 2). China used to have the lowest heart disease rate in the world; it also enjoyed the cleanest air quality. In concert with China’s industrial revolution we see an increase in the well-known dirty coal-based fuel electric plants.  This only serves to exacerbate the issues.  In (Figures 4 and 5) one notes a direct correlation between increased

Figure 5. Deaths and Pollution.

fossil fuel use in industrialization with an increased mortality rate as well as an increase in heart disease.  The industrial revolution in China is being driven by the conversion of fossil fuels to energy just like here in the U.S.  The use of fossil fuels has led to several environmental problems such as reduction in our carbon energy sources, water pollution, habitat destruction, air pollution, heart disease and an energy crisis in the poorer countries around the world.  Meanwhile, CO2 levels are still climbing and the greenhouse effect is growing as a function of burnt fuel along with increased acid rain which will no doubt create new problems (Durairai et al, 2012).

Problem Summation.

To recap, by looking at China and the EPA we see that we must move from fossil fuels to reduce negative emission, climate change, poor air quality, and to save lives.  It is recommended that we use a temporary solution that will lower emissions while not causing major disruptions in the current energy systems of society nor threaten the bottom dollar of those who supply fossil fuels.  For this reason, HHO will be introduced as an additive to fossil fuel that will at least temporarily provide some reduction in emission until a more permanent solution can be obtained.  To understand the basics of the technologies included in this paper, please refer to Appendix (L). The Stakeholders of this problem are all human life, anyone profiting from fossil fuel consumption, anyone concerned about public health, anyone concerned about energy availability, anyone in the energy transportation industry, and all nations and peoples that use power, electricity, and transportation. Also included are nations that depend on the production of fossil fuels such as the Arabian Peninsula or other such Middle Eastern countries. According to the International Energy Agency (IEA), these countries account for 66 % of the world’s fossil fuels. Countries such as Saudi Arabia (13%), Russia (13%), United States (12%), are among the highest that may be affected (IEA, 2014). For more info on the stakeholders and those in control of fossil fuels please refer to Appendix (L) background--stakeholders.

Literature Review

The papers in the following literature review offer answers to the many initial hurdles   In reviewing articles in this literature I am looking for answers to such questions as the following:  1. Can H2 burn in an engine? 2. Is it real--can one produce a burnable form of H2 from water easily and cheaply? 3. Does HHO have significant data supporting the decrease of auto emission to warrant further study? 4. Is it true that one can build and HHO cell at little cost?

H2 burns in an engine.

NASA in 1977 did an experiment using a multi-cylinder engine to extend the efficient lean operating range of gasoline by adding hydrogen. However, Lee and Brehob indicated slightly increased hydrocarbon emissions as well (Lee and Brehob, 1971).	The results indicated that flame speed has increased significantly. In leaner mixtures mixing hydrogen with gasoline reduced emissions.  NOx was reduced by a factor of 19; however, when engine performance reduced NOx, values actually increased. Further, at an idle, Hydrocarbons were slightly higher.  On the other hand, Carbon Monoxide amounts were reduced.  This article shows that hydrogen burns in today’s engines as a drop-in replacement for fossil fuels.  It should be noted that JPL conducted similar experiments with the same results.  Please refer to the data in the graphs. The conclusion of the review was positive; H2 can burn in an engine (Lee and Brehob, 1971).

Rudolf A. Erren made hydrogen-fueled engines in the 1920s including trucks and buses. Allies of World War II found a submarine and even torpedoes converted by Erren to hydrogen power proving the government has already used this technology (Erren and Campbell, 1933).  Another article reported Hydrogen and Oxygen have been used as a fuel for the submarine since the end of the world wars. The main reason for this is the fact that drinkable water is its exhaust. This shows again this technology has already been used. (King and Rand, 1955).  Robert Zweig converted a pickup truck to run on H2. It still operates and can be seen at The American Hydrogen Association where it is displayed in public exhibits (Zweig, 1992). This auto shows that this technology can be used for any common automobile.

The HHO cell and emissions.

Stebar and Parks investigated the effects of hydrogen-supplementation.  Very low NOx and CO emissions were achieved. Also, significant thermal efficiency improvements resulted from the extension beyond isooctane lean limit operation (Stebar and Parks, 1974).  Another study showed that the hydrogen–diesel co-fueling solved the drawback of lean operation of hydrocarbon fuels such as diesel.  This reduced misfires, improved emissions, performance and fuel economy (Masood et al., 2006).  Ji and Wang studied hydrogen–gasoline mixtures in engines and concluded that wide flammability and fast burning velocity of hydrogen yielded reduced CO and HC emissions at idle and lean conditions (Wang, Ji 2009). Rajeshkumar in his paper introduced four methods to reduce the exhaust emissions. The result showed fossil fuel was much nearer to complete combustion which in turn ensures that there are no unburned hydrocarbons. Oxidation of the partially oxidized carbon i.e. CO was also more completely burned into CO2 (Rajeshkumar et al, 2016).  However, Jingding studies have shown that if mixtures are made lean and spark timing is retarded NOx can be reduced below the current standards for emissions (Jingding, 1998).  Researchers at Mutah University showed that a mixture of HHO, air, and gasoline cause a reduction in emission pollutants and enhancement in engine efficiency.  Emission of NO and NOx were reduced by an astounding 50%. Moreover, carbon monoxide concentrations were reduced by 20% while fuel consumption was reduced by 20-30% (Yimaz et al, 2010).  Figures 6-9 are graphs of the results of experiments by Yilmaz et al.

Figure 6.

Figure 7.

Figure 9

Figure 8.

HHO at little cost.

Cunningham et al made researches on methods and apparatus for enhancing combustion in an ICE through electrolysis and produced hydrogen along with oxygen yielding an enhanced combustion at low engine loads for all types of engines with little cost (Cunningham et al, 1992).  Ammar A presents work in the design of a device attached to the engine to integrate an HHO production system with the gasoline engine. He states this device can be easily installed in any engine compartment. In our experiments, we were able to manufacture a working HHO cell for less than twenty dollars (Morse, unpublished manuscript, 2016).

HHO with Diesel and Gasoline.

Durairai wrote a paper dealing with bio-diesel and HHO gas from hydrolysis. This report states that the using of water powered biodiesel results in a substantial reduction in emission (Durairai et al, 2012).  Yilmaz (2010) injected small rates of HHO instead of H2 into a diesel engine. Experiments performed showed positive results in improving the fuel efficiency of the engines. This was true for fossil diesel as well as Bio-Diesel fuels in engines.

Arguments and Discussions in the Literature Review.

The NASA experiments showed an increase in NOx due to the fact that H2 was burned alone. However since HHO has O2 this problem has been overcome. Studies show that concentrations of nitrogen oxide have been reduced by up to 50% or more by Yilmaz et al. A problem with temperature was reported in most of the articles; however, this has been overcome by the inclusion of an electrolyte (baking soda) which is cheap and commonly available and by changing from a continuous direct current to a pulsed one (Yimaz et al, 2010)(Morse, 2016). It seems the main problem with this technology is the amount of electricity it seems to take to make it have a net gain. A”No financial feasibility” given by Cameron was concluded when in his experiments the net gain was less than the electrical cost of production (Cameron, 2012).  These problems were addressed in several experiments.  First two different researchers have shown that HHO can reduce diesel consumption (Yilmaz, 2010). Also it has been found that the Cameron experiments operated at the highest current cost voltage range. It was found in our experiments that the operating voltages of auto charging systems are in the worst possible range for current usage.  Bambagn (2016) shows that the answers to both the current usage issue as well as part of the heat issues are overcome by pulse or frequency generation. Please refer to Appendix (L) for more information on this (Bambagn et al, 2016). To add to this our studies found it possible to use a variable frequency wave and drop the current draw to 0 amps.  This means it is possible to produce HHO with very little electrical current draw.  (Figure 10) shows that as voltage increases heat production increases. (Figure 11) shows that is an area of gas production with less current draw. (Figure 12) shows that gas production increases with voltage. (Figure 13) shows that current has a bell-shaped curve, this means that as voltage increases currents consumption drops.

As stated above it was found that the greatest draw on a power source is from 12 to 16 volts which is what the range is for the standard auto charging system. (Figure 13) indicates that at some voltage greater than 20 volts it may be possible to have a low current drain.

Finally, the graph (Figure 14), shows that at a frequency of about 55khz electric current draw drops to 0 amp.  For Results from Voltage Versus Amp experiments and review please consult the Appendix (A-F).

Literature Review Summation.

Many of the articles reviews were able to overcome many of the obstacles that stood in the way of this technology being implemented. The main obstacles that we reviewed were as follows.

- Yes H2 and HHO can be burned in an engine. Erren and Campbell in 1920 were successful.

- Yes HHO can be produced from electrolysis for little cost (Yull Brown patent, in 1977).

- Though H2 alone is not suitable as an alternative fuel source for a drop-in replacement, HHO with the water content will reduce all emissions and enhance power and torque (Lee and Brehob, 1971).

Hypothesis

In this study and review, we looked at one of the enduring alternative additives to gasoline and diesel. We have reviewed many of the arguments and solutions to those arguments. It is hypothesized that since it is possible, affordable and easy to implement the use of HHO as a means of reducing emissions that such technology would show promising emission reductions in a state such as New York. The evidence is as follows: NASA showed H2 burns in an engine, Stebar, and Parks proved using HHO has low emissions, Masood proved the same is true for CI engines, Researchers at Mutah University showed reduced emissions and enhanced engine efficiency, and Cunningham produced and HHO cell at little cost. Finally our studies back up these findings in our unpublished manuscript (Morse, 2016).

Methods

The methods for this paper were very difficult as problems occurred in the interpretation of instructions. First the “Methods and Applications Summative Assessment” was read and reviewed. Out of the given topics “Reduced Carbon Emissions for Businesses in X state” was chosen. The next step was to find some current technology that was on the forefront of science that would reduce emissions.  After an extensive search on scholarly sites several papers and articles about HHO were found. While such sites as the Leatherby Libraries and Goggle Scholar were heavily used a semi-complete list of the sites that were further reference are listed in Appendix (M).  Search parameters included “China Energy” and “HHO” as well as many others. After reviewing all the papers and articles cleanly (without annotation) a re-read was performed extracting information and arguments in support or against. An effort was then made to find data tables from the links given in the “Methods and Applications Summative Assessment” instructions. Unfortunately this was almost fruitless as most links had no available data tables. However, one did, the New York website; albeit this site only provided data on emission totals for the state.  It was suggested in the instruction to employ a government site data so a search for any government site that dealt with emissions was performed with results on the Environmental Protection Agency site. The mission of EPA is to protect human health and the environment. Its purposes include such things as ensuring that all Americans are protected from significant risks to human health and the environment where they live, learn and work; additionally it works to enforce policies concerning natural resources, human health, economic growth, energy, transportation, agriculture, industry, and international trade.  These factors are similarly considered in establishing environmental policy and contribute to making communities and ecosystems diverse, sustainable and economically productive. The EPA was established Dec. 2, 1970 by then president Richard Nixon by executive order.   Data from the EPA concerning emissions was used to clarify the different forms of emission (Please see Appendix ‘L’, or the EPA website). Further other data concerning the growth of China and the environmental impact of using fossil fuels was cited from greenpeace.org and other sites. Greenpeace is an independent global campaigning organization. It acts to change attitudes and behaviors, to protect and conserve the environment and to promote peace by catalyzing an energy revolution to address climate change and to create a toxic-free future with safer alternatives to forms of fuel that produce an emission that is hazardous to life.  It was established in 1971 in Vancouver Canada. A final web data source was (ny.gov) the official website of the state of New York.  After reading the article we identified any problem areas that were not answered. We constructed an experiment which verified the current findings in the literature review, as well as sought for a solution to net current consumption, the last remaining obstacle. The experiment took several weeks and about one hundred hours. Special equipment was purchased to take measurements of wave pulse, temperature, and frequencies. The data from these experiments are available in the Appendix (A-F).  Once all barriers were overcome the final step was to use the data from the New York state site in tandem with the data from the possible reduction available by employing the HHO technologies to show possible net profits and gains in the reduction of emissions.

Final Analysis and Discussion

HHO has been looked at as an alternative additive to fossil fuels to help relieve the stresses caused by the use of such fuels as well as hardships caused by the limitation of supply.  It was found that in the NASA study that H2 can burn in engines. It has further been shown in the Yilmaz literature review that HHO does significantly reduce all auto emissions while reducing fuel consumption. 	As a result, a study was performed by varying the frequency and wavelength in an attempt to find optimal voltages and frequencies.  By reviewing the data tables in Appendix (A-F) it was found that the power requirements can be much reduced by using voltages above 30 volts; further, by employing a full wave rectifier or a pulse, it was found to have a cell produce efficient amounts of HHO while having a 0 amp draw. Please refer to the graph (Figure 14). Having removed all arguments against the HHO cell as a viable means of reducing emissions we looked at the possible outcome of using such technology.  A review of data from a data table given by the New York State website reveals that trends show that in the years 1990-2012, transportation has been by far the greatest contributor of emissions in New York. (Figure 15) is a graph that was created from the data tables in Appendix (G) to show just how much more of a contributor transportation was compared to other sources of emissions.  We see that transportation drawn in (red) as over double the amount until in 2012 it was matched by electric production from fossil fuels.  Using the data from Buffalo N.Y. (Appendix G) listed by the state website we can compare the possible reduction of emissions that might be obtained by implementation of the HHO cell in autos.  The state data listed in Appendix (G) was compared with the Cameron Data listed in Appendix (H) for these graphs.  Figures 16-19 show the possible reduction of SO2, NOx, CO2 and CO. These are substantial as this is enough to save lives by the reduction of emissions -- to gain forward momentum in the war against climate change and global warming while having little effect on the bottom dollar of those who provide fossil fuels. Additionally, it is believed that reductions in emissions from implementing this technology in fossil burning electric plants might very well produce similar results.  All this can be done with little disturbance to the current fossil fuel energy consuming societies. By implementing this technology immediately we can see an abrupt net gain reducing the problems of emission, smog, and global warming.

Final Conclusions and Recommendations

Conclusions

We have found that H2 burn in an engine. We have further found that we can one produce a burnable form of H2 from water easily using electrolysis.  Further, HHO has significant data supporting the decrease of auto emission to continue experimentation and implementation. We have successfully built a cell for less than 20 dollars.  We have found that any voltage above 30 volts, the higher the better, is sufficient to produce HHO with an electrolyte present. Through manipulation of the waveform and frequency, we found it possible to operate an HHO cell efficiently at a zero amp current draw. Continuing, evidence supports that implementation will reduce emissions in major cities such as Buffalo N.Y.  as transportation was found to be the highest contributor to the emission problem in urban areas. Lastly, a comparison of the data from Buffalo N.Y. with the data from the Cameron 2012 provided evidence that a significant reduction of emissions is possible.

Recommendations

While it is recommended to continue experimentation using PWMs and high voltages along with design modifications for the greatest possible production potential. Further, cell design experimentation using a fully rectified D.C. pulsed wave should also continue.  It is also recommended that this technology should be distributed and used as soon as possible.  This technology can be produced cheaply, and has such great potential for immediate reduction of emissions and known greenhouse gasses that this technology should move to the forefront with governmental implementation on governmentally run transportations. However, it is felt that using water as a source of fuel is not a good idea as it is the lifeblood of everything on the planet and thus this technology is only recommended for use in the short term.

References

Adams, Walter (1968) The American Economic Review. Vol. 58, No. 2, Papers and Proceedings of the 	Eightieth Annual Meeting of the American Economic Association (May, 1968), pp. 652-665

Published by: ( URL: http://www.jstor.org/stable/1831851

Adnan, R.; Masjuki, H.; Mahlia, T. (2009) An experimental investigation of unmodified DI diesel engine 	with hydrogen addition. 2009, pp. 45-49.

Alias A. (2005) Two Days National Seminar on Alternative Energy Resources. August, 27-28, Thane, India.

Ammar A. Al-Rouson (2010) Reduction of fuel consumption is gasoline engines by introducing HHO gas 	into intake manifold. Department of Mechanical Engineering, Faculty of Engineering, Mutah 	University Mutah, Al-Karuk 61710 Jordan. 10 March 2010.

Antonia V.; Herzog, Timothy E.; Lipman, Daniel M. (2001) Sources Report. Encyclopedia of Life Support 	Systems Publication. University of California, Berkeley USA, 63 p. Laboratory (RAEL),

URL: http://socrates.berkeley.edu/~rael

Bacon, F.T. (1960) The high-pressure hydrogen-oxygen cell. J Ind Eng Chem 1960;52(4):301-3

Bambagn, Sudaramanta; Sudjud, Darsopuspito; Djoko, Sungkono (2016) Fuels, and Combustion 	Engineering Laboratory. Department of Mechanical Engineering, Sepuluh Nopember Institute of 	Technology, Surabaya, Indonesia.

Bambagn, Sudaramanta; Sudjud, Darsopuspito, Djoko, Sungkono (2016) Application of dry cell HHO gas 	generator with pulse width modulation on spark ignition engine performance.  Fuels and 	Combustion Engineering Laboratory, Department of Mechanical Engineering, Sepuluh 	November Institute of Technology, Surabaya, Indonesia. Ijret International Journal of Research 	in Engineering and Technology, 2016.

Bari, S.; Esmaeil, M. M. (2010) Effect of H2/O2 addition in increasing the thermal efficiency of a diesel 	engine. Fuel, vol. 89, pp. 378-383, 2010.

Bell S.R.; Gupta, M. (1997) Extension of The Lean Operating Limit For Natural Gas Fueling of a Spark 	Ignited Engine Using Hydrogen Blending. Combustion Science and Technology, 123:23–48.

Benkataraman, B.; Elangoe, D. (1998) Renewable Energy Sources. Hindustan College of Engineering, 	Padur, India, 235 p.

Bhavesh, V.; Chauhan, M.E.; Rathod, Gaurav P.; Patel, Tushar M. (2016) An Experimental Investigation of 	HHO Gas and Varying Compression Ratio on Emission Characteristics of Constant Speed Diesel 	Engine (CI). Indian Journal of Research Issue 4 April 2016 Patel Associate professors, L.D.R.P-ITR, 	Near KHA-5 Circle, Sector-15, Gandhinagar.

Bhardwaj. S.; Verma, A.S.; Sharma, S.K. (2014) Effect of Browns gas on the performance of a four-stroke 	gasoline engine. Int. Journal of Emerging Technology and Advanced Engineering (Online). 	Volume 4, Special Issue 1 Feb 2014

Birtas, A.; Voicu,  I; Petcu, C.; Chiriac, R.; Apostolescu,  N. (2011) The effect of HRG gas addition on diesel 	engine combustion characteristics and exhaust emissions. International Journal of Hydrogen 	Energy, vol. 36, pp. 12007-12014, 2011

Boretti, A.(2010) Comparison of Fuel economies of high-efficiency diesel and hydrogen engines powering 	a compact car with a flywheel-based kinetic energy recovery system, Int. Journal of Hydrogen 	Energy 35 (2010) 8417-8424

Bose, P.K.; Maji, D. (2009) An experimental investigation on engine performance and emissions of a 	single cylinder diesel engine using hydrogen as inducted fuel and diesel as injected fuel with 	exhaust gas recirculation. International Journal of Hydrogen Energy, vol. 34, pp. 4847-4854, 	2009.

Bowman, C.T. (1992) Control of combustion-generated nitrogen oxide emissions: technology driven by 	regulation.  1992, pp. 859-878.

Brown, Yull. (1977), A technique to generate HHO gas based on electrolysis process. Brown Y. Brown’s 	gas. United States Patent. US Patent 4014,777; March 28, 1978

Bureau of Agriculture, (2011) Energy in Australia 2011.  D. of and E. a. T. Resources, Eds., ed. Canberra: 	Australian Bureau of Agriculture and Resource Economics and Sciences, 2011.

Cameron, R. (2012) Surveying Effects of Onboard HHO and Water Injection in a Diesel Generator. A 	dissertation submitted by Rick Cameron, University of Southern Queensland Faculty of 	Engineering and Submitted: October 2012.

Cunningham, J.E.; Goodenough, R. (1992) Method and Apparatus for Enhancing Combustion in an 	Internal Combustion Engine through Electrolysis. United States Patent, April, 21, 1992.

Das, L.M. (2002) Near-Term Introduction of Hydrogen Engines For Automotive and Agriculture 	Application. International Journal of Hydrogen Energy, 27:479–87.

Das, L.M. (1991) Exhaust Emission Characterization of Hydrogen Operated Engine System: Nature of 	Pollutants and Their Control Techniques. International Journal of Hydrogen Energy, 16:765–75.

Dunn, S. (2002) Hydrogen futures toward a sustainable energy system. Int. Journal of Hydrogen Energy 	2002;27:235-64

Durairai, R.B.; Shanker, J.; Silvasankar. M. (2012) HHO Gas with Bio Diesel as a Dual Fuel with Air 	Preheating Technology. SciVerse ScienceDirect. Procedia Engineering 38 2012 1112-1119 Faculty 	of Engineering and Technology, SRM University, Chennai India. Professor and Director Center for 	BioMedical Research, Arunai engineering college, Thirubanamalai, India. 2012 published by 	Elsevier LTD. Selection and/or peer-review under responsibility of Noorul Islam Centre for 	Higher Education

Eckman, C. (2006) Plasma orbital expansion of the electrons in water. Proc. NPA, vol. 7, pp. 142-144, 	2010.

EPA, Environmental protection agency (1994) EPA 400-f-92-007 Fact Sheet OMS-5 August, 1994

Automobile Emissions: An Overview.

Erickson, Peter; Lazarus, Michael (2016) How would phasing out U.S. federal leases for fossil fuel 	extraction affect CO2 emission and 2°C goals. Stockholm environment institute working paper 	no. 2016-12 Stockholm Environment Institute – U.S. Center.

Erickson P.; Lazarus, M.; Tempest, K. (2015) Carbon Lock-in from Fossil Fuel Supply Infrastructure. SEI 	discussion brief. Stockholm Environment Institute. Seattle, WA, US.

http://www.sei-internationsl.org/publications?pid=2805

Erren, R. A.; Campbell, W.H. (1933) Hydrogen: A Commercial Fuel for Internal Combustion Engines and 	Other Purposes. Journal of Inst. Fuel. 6:277-90.

Fanhua Ma; Mingyue Wang. (2010) Performance and emission characteristic of a turbocharged spark-	ignition hydrogen-enriched compressed natural gas with high hydrogen ratio. Int. Journal of 	Hydrogen Energy 2010;356438-47

Ghiffari. Y.A.; Sungkono, D. (2013) Study of HHO Gas Generator Characteristics Wet Cry Cell and Cell type 	with dimensions of 80x 80 mm with the addition of PWM E-3 FF (1 kHz). Pomits engineering 	Journal Vol. 1 No. 1. (2013).

Harper, D. (2007) Online Etymology Dictionary. (Access date: 12 April 2010)

http://www.etymonline.com/index.php?search=energy&searchmode=none

Hekkert, M.P.; Van-Giessel, J.F.;,Ros, M.; Wietschel, M.; Meeus, M.T.H. (2005) The evolution of hydrogen 	research: is Germany heading for an early lock-in? Int. Journal of Hydrogen Energy 	2005;30:1045-52

Herzog, A.V.; Lipman T. E., Kammen D. M.; (2006) Renewable Energy

Haywood, J. B. (1998) Internal Combustion Engine Fundamentals. McGraw-Hill Publication. 	Massachusetts Institute of Technology, Boston, USA. 15-371.

Hoehn, F. W.; Parks, M. W. (1974) Feasibility Demonstration of a Road Vehicle Fueled with 	Hydrogen-Enriched Gasoline, Ninth Intersociety Energy Conversion Engineering Conference. Am, 	Soc. Mech. Eng., 1974, pp.  956-964

Hoehn, R. W.; Baisley, R. L.; and Dowdy, M. W. (1975) Advantages in Ultra-lean Combustion Technology 	Using Hydrogen-Enriched Gasoline. Tenth Intersociety Energy Conversion Engineering 	Conference.  Inst. Electric. Electron. Eng., 1975, pp 1156-1164

Ijret International (2015) Application of dry cell HHO gas generator with pulse width modulation on spark 	ignition engine performance. Journal of Research in Engineering and Technology

Jingding, L.; Linsong, G.; Tianshen, D. (1998) Formation and Restraint of Toxic Emissions in Hydrogen–	gasoline Mixture Fueled Engines. International Journal of Hydrogen Energy, 23(10):971-5.

Kahraman, E. (2005) Analysis of a Hydrogen-Fueled Internal Combustion Engine. The Graduate School of 	Engineering and Sciences of İzmir Institute of Technology. M.Sc. Thesis 71 p.

Keith, Wayne (2012) Wood Gas Video Series. https://www.youtube.com/watch?v=_tA0stq9rjg

King, R. O.; Rand, M. (1955) The Hydrogen Engine. Canadian Journal of Technology.

Krawczyk, T. (1996) Biodiesel– Alternative Fuel Makes Inroads but Hurdles Remain. American Oil 	Chemists Society Second Edition.California, USA,

Lee, R. C. Brehob, W. (1971) Effect of compression Ratio, Mixture Strength, Spark Timing, and Coolant 	Temperature Upon Exhaust Emissions and Power.  SAE Paper 710832, Oct. 1971 NASA

Lee, R. C. Brehob, W. (1971) Mechanisms of Pollutant Formation and Control from Automotive Sources. 	SAE Paper 710483, June 1971 NASA

Lilik, G.K.; Zhang, H.J.; Herreros, M.D.; Haworth, C.;  Boehman, A. L. (2010) Hydrogen assisted diesel 	combustion. International Journal of Hydrogen Energy, vol. 35, pp. 4382-4398, 2010.

Masood, M.; Ishraat, M. M.; Reddy, A. S. (2007) Computational Combustion and Emission Analysis of 	Hydrogen–Diesel Blends with Experimental Verification. International Journal of Hydrogen 	Energy,32:2539-47.

Masood, M.; Isharat, M.M.; Reddy, A.S. (2006) Computational Combustion and Emission Analysis of 	Hydrogen–Diesel Blends with Experimental Verification. International Journal of Hydrogen 	Energy, 32:2548-52.

MCcarthy J.  (2008) Hydrogen Study. Stanford University. (Access date: 10 March, 2010)     	http://wwwformal.stanford.edu/jmc/progress/hydrogen.html

Medhursta, Martin J. (2009) The Military-Industrial Complex and the New Industrial State.  Re-	conceptualizing rhetorical history: Eisenhower's farewell address pages 195-218, Publishing 	models and article dates explained Published online: 05 Jun 2009.

Michrowski, A. (1993) Anomalous Water Explained by Brown's Gas Research. Planetary Association for 	Clean Energy Newsletter. 6 (4):10-11. 74 -- 1993. Yull Brown’s Gas. Planet. Assoc. Clean Energy 	News, 6 (4):10- 11.

Millman, J.; Grabel, A. (1987) Microelectronics. Second Edition. McGraw-Hill Inc. Publication. New York, 	USA. 690 p.

Mohamed, M.; Kassaby, EL.; Yehia, A.; Eldrainy, Mohamed E.; Khidr, Kareen; Khidr, I.(2015) Effect of 	hydroxy (HHO) gas addition on gasoline engine performance and emissions. Alexandria 	University, Alexandria Engineering Journal. Mechanical Engineering Department, Faculty of 	Engineering, Alexandria University, Egypt. 8 June 2015. Faculty of Engineering, Alexandria 	University. Production and hosting by Elsevier B.V. This is an open access article under the CC 	BY-NC-ND License.

Momirlan ,M.; Veziroglu, TN. (2002) Current status of hydrogen energy. Renewable Sustainable Energy 	Rev. 2002;6:141-79

Morse, J (2016) HHO A Viable fossil fuel additive HHO viable fossil fuel

Musmar, Sa’ed A.; Ammar A. Al-Rousan. (2011) Effect of HHO gas on combustion emission in gasoline 	engines. Department of Mechanical Engineering , Faculty of Engineering, Mutah University, 	Mutah, Al-Karak 61710, Jordan  16 Feb 2011

Pogue, (1962) US Patent Pogue carb, U.S. Pat. 2,025,798. CARBURETOR Filed Sept. 2'7, 1935

https://www.google.com/patents/US3049850

Rajaram, S. (2014) Effectiveness of oxygen enriched hydrogen-HHO gas addition on direct injection diesel 	engine performance, emission, and combustion characteristics. 18(1), 259-268.

Rajeshkumar G.; Rajapandian R.; Surendrakumar G.; Anathakumar P.; Vasanthanseelan S.; Senthil 	Ganesh A. (2016) Experimental investigation of emission control in IC Engines By introducing 	various techniques. Engineering & Technology in India www.engineeringand 	technologyinindia.com Vol 1:2 March 2016

Shell; Richard, Michael Graham (2008) 350mpg Shell Car. (Online).

http://www.treehugger.com/cars/souped-down-1959-opel-t-1-gets-37659-mpg.html

Rogelj, J,; Schaeffer. M.; Meinshausen,M.; Knutti, R.; Alsomo, J.; Riahi, K.; and Hare, W. (2015). Zero 	emission targets as long-term global goals for climate protection. Environmental Research 	Letters. 10(10). 105007. DOI:10.1088/1748-9326/10/10/105007.

Rystad, Energy (2015). UCube, Version 1.18 Oslo. Norway. 	http://www.rystadenergy.com/Databases/UCube. Media

Sankar, T. (2014) The effect of oxy-hydrogen (HHO) on the performance and emission characteristics of 	diesel and karanj in single cylinder four stroke diesel engine. 2(1), 12-18

Santilli, R.M. (2006) A new gaseous and combustible form of water. International Journal of Hydrogen 	Energy, vol. 31, pp. 1113-1128, 2006.

Saravanan, N.; Nagarajan, G.  (2008) An Experimental Investigation of Hydrogen-Enriched Air Induction in 	a Diesel Engine System. International Journal of Hydrogen Energy.

Schulti, I.; Hart, D.; Vorst, R. (2004) Issues affecting the acceptance of hydrogen fuel. Int. Journal of 	Hydrogen Energy, 2004;29:677-85

Sierens, R; Rosseel, E. (1995) Sequential injection of gaseous fuels. In: proceeding of the 5th 	International congress: the European automotive industry meets the challenges of the year 	2000; 1995 June 21-23. Strasbourg: EAEC Congress; 1995. P. SIA 9506A03

Sierens, T.; Rosseel, E. (1998) Variable composition hydrogen/natural gas mixtures for increased engine 	efficiency and decreased emissions. In: proceeding of the spring engine technology conference; 	FortLauderadale; 1998 April 26-29. ASME; 1998. P. 98-ICE-105.

Stebar, R.F.; Parks, F.B. (1974) Emission Control with Lean Operation Using Hydrogen-Supplemented 	Fuel. SAE Paper (Document No.740187).

Tauzia, X.; Maiboom, A.; Shah, S.R. (2010) Experimental study of inlet manifold water injection on 	combustion and emissions of an automotive direct injection Diesel engine. Energy, vol. 35, pp. 	3628-3639, 2010.

Tekin, M.; Cavusoglu, Y. (1997) Pollutant Formation and Its Control in Internal Combustion Engines. 1st 	Automotive Technology Congress with International Participation, May, 26-30, Adana, Turkey.

Tesla, N. (1889) Thermo-Magnetic Motor. Patent 396,121, Specification forming part of Letters Patent 	No. 396,121, dated January 15, 1889. Application filed March 30, 1886. Serial No. 197,115.

Vader, N.V.; Joshi, V.A. (2005) Wind Power Generation Technology. Two Days National Seminar on 	Alternative Energy Resources. August, 27-28, Thane, India.

Venkataraman, B.; Elango, D. (1998) Renewable Energy Sources, Hudustan College of Engg, Padur-603 	103 Tamil Nadu

Verhelst, S.; Wallner, T. (2009) Hydrogen-fueled internal combustion engines. Progress in Energy and 	Combustion Science 35 (2009) 490-527

Walsh, M. P. (2001) Global Trends in Diesel Emissions Regulation-A 2001 Update. SAE paper, pp. 01-	0183, 2001.

Wang, S.; Ji, C.; Zhang, B.; Liu, X. (2014) Realizing the part load control of a hydrogen-blended gasoline 	engine at the wide open throttle condition. Int. Journal of Hydrogen Energy 39(2014) 7428-7436

Wang, S.; Ji, C. (2012) Cyclic variation in a hydrogen-enriched spark-ignition gasoline engine under 	various operation conditions. Int. Journal of Hydrogen Energy 37 (2012) 1112-1119

Wang, S.; Ji, C.; Zhang, B.; Liu, X. (2012) Performance of a hydro-oxygen-blended gasoline engine at 	different hydrogen volume fractions in the hydro-oxygen. Int. Journal of Hydrogen Energy 37 	(2012) 13209-13218

Wang, S.; Ji, C.; Zhang, B. (2011) Starting a spark-ignited engine with the gasoline-hydrogen mixture. Int. 	Journal of Hydrogen Energy 36 (2011) 4461-4498

Wang, S.; Ji, C.; Zhang, B.; Zhang, J. (2011) Comparison of the performance of a spark-ignited gasoline 	engine blended with hydrogen and hydrogen-oxygen mixtures, Energy 36 (2011) 5832-5837

Wang, S.; Ji, C.; Zhang, B.; Zhang, J. (2011) Improving the performance of a gasoline engine with the 	addition of hydrogen-oxygen mixtures. Int. Journal of Hydrogen Energy 36(2011) 11164-11173

Wang, S.; Ji, C.; Zhang, B. (2010) Effect of spark timing on the performance of a hybrid hydrogen-gasoline 	engine at lean conditions, Int. Journal of Hydrogen Energy 35 (2010) 2203-2012.

Wang, S.; Ji, C. (2010) Experimental study on combustion and emissions performance of a hybrid 	hydrogen-gasoline engine at lean burn limits. Int. Journal of Hydrogen Energy 35 (2010) 1453-	1462

Wang, S.; Ji, C. (2009) Combustion And Emissions Performance of a Hybrid Hydrogen–Gasoline Engine at 	Idle and Lean Conditions. International Journal of Hydrogen Energy, 35(1): 346-55.

Wang, S.; Ji, C. (2009) Effect of hydrogen addition on combustion and emissions performance of a spark 	ignition gasoline engine at lean conditions. Int. Journal of Hydrogen Energy 34 (2009) 7823-7834

White, C.M.; Steeper, T.T.; Lutz, A.E. (2006) The hydrogen-fueled internal combustion engine: a technical 	review, Int. J. Hydrogen Energy 31 (2006) 1292-1305

Widhiyanuriyawan, Denny (2016) 325999: All other miscellaneous chemical product manufacturing.

325998: All Other Miscellaneous Chemical Product and Preparation Manufacturing. 221210: 	Natural Gas Distribution. 325181: Alkali and chlorine manufacturing. 325180: Other Basic 	Inorganic Chemical Manufacturing. 325189: All other basic inorganic chemical manufacturing. 	Applied Mechanics & Materials; 2016, Vol. 836, p294-298, 5p (2016).

Wyer, Samuel S. (1906) A Treatise on Producer- gas and Gas producers. American society mechanical 	engineers, Library of the University of California, Hill Publishing Company New York and London, 	The Engineering and Mining Journal, Power, American Machinist. 1907 	http://www.driveonwood.com/static/media/uploads/pdf/treatise_on_producer_gas_and_gas_	producers.pdf

Yilmaz, A.C.; Ulundamar, E; Aydin, K (2010) Effect of Hydroxy (HHO) gas addition on performance and 	exhaust emissions in compression ignition engines. Int. Journal of Hydrogen Energy 	2010;35:11366-72  http://doi.org/10.1016/j.jihydene.2010.07.040

Yilmaz, Ali Can (2010) Design and application of Hydroxy HHO system. Cukurova University Institute of 	Natural and applied sciences  MSc Thesis  Ali Can Yilmaz. Department of Mechanical Engineering

Adana, 2010 12-07-2010.

Yilmaz, A. C.; Uludamar, E.; Aydin, K. (2010) Effect of hydroxy (HHO) gas addition on performance and 	exhaust emissions in compression ignition engines.  International Journal of Hydrogen Energy, 	vol. 35, pp. 11366-11372, 2010.

Zeng K.; Zhang, D. (2010) Recent progress in alkaline water electrolysis for hydrogen production and 	applications.  Progress in Energy and Combustion Science, vol. 36, pp. 307-326, 2010.

Zweig, R. M. (1992) Proceedings of the Ninth World Hydrogen Energy Conference. October, 8-12, Paris, 	France.

Data Sources

http://folk.uio.no/glen Graphs and data for Emission is China

EPA.gov for Data concerning EPA

https://data.ny.gov For Emission Data in New York

http://jefferymorse.net for Data on my experiment.

https://www.iea.org/statistics/ For data on energy systems of the world

Appendix

Appendix A.

D.C. Voltages in distilled water.

| d.c. | Volts | amps | ml | by sight | minutes | delta T |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 | 0 | 0 | no | 0 | 0 |  |
|  | 5 | 0 | 0 | no | 0 | 0 |  |
|  | 9 | 0 | 0 | no | 0 | 0 |  |
|  | 12 | 0 | 0 | no | 0 | 0 |  |
|  | 15 | 0 | 0 | no | 0 | 0 |  |
|  | 20 | 0 | 0 | no | 0 | 0 |  |
|  | 112 | 0 | 0 | no | 0 | 0 |  |
|  | 190 | 0.02 | 2 | yes | 20 | 2 |  |
|  | 2100 | 0.25 | 30 | yes | 2 | 10 |  |

Appendix B.

D.C Volt versus Amps Comparison.

Volts:amps

0:0 		4.6:0.02	4.7:0.03	5:0.04		5.3:0.05	5.8:0.06

5.9:0.07	6.2:0.08	6.4:0.09	6.6:0.1		6.8:0.11	7:0.12

7.3:0.13	7.4:0.14	7.6:0.15	7.8:0.16	8:0.17		8.3:0.18

8.5:0.19	8.7:0.2		8.9:0.21	9:0.22		9.2:0.23	9.5:0.24

9.7:0.25	9.9:0.26	10.1:0.27	10.3:0.28	10.5:0.29	10.6:0.3

10.8:0.31	11:0.32		11.2:0.33	11.4:0.34	11.6:0.35	11.8:0.36

12:0.37		12.2:0.38	12.4:0.39	12.6:0.4 	12.8:0.41	13:0.42

13.1:0.42	13.2:0.43	13.3:0.43	13.4:0.43	13.5:0.43	13.6:0.44

13.7:0.44	13.8:0.44	13.9:0.44	14:0.44		14.1:0.44	14.2:0.44

14.3:0.44	14.4:0.44	14.5:0.44	14.6:0.43	14.7:0.43	14.8:0.43

14.9:0.43	15:0.42		15.1:0.42	15.2:0.41	15.3:0.41	15.4:0.41

15.5:0.4	15.6:0.39	15.7:0.39	15.8:0.38	15.9:0.38	16:0.38

16.1:0.37	16.2:0.35	16.3:0.35	16.4:0.33	16.5:0.31	16.6:0.28

16.7:0.27	16.8:0.27	16.9:0.27	17:0.26		17.1:0.24	17.2:0.23

17.3:0.2	17.4:0.19

Appendix C.

Audio Signal voltage and amperage with distilled water.

Hz  Amp ma   Volts	Hz  Amp ma   Volts	Hz  Amp ma   Volts	Hz  Amp ma   Volts

20    27.7     	0.2	30	30	0.3	40	25.7	0.4	50	27.8	0.4

60	35.3	0.4	70	39.7	0.4	80	42.5	0.4	90	44.7	0.4

100	46.9	0.4	110	48.8	0.4	120	50.5	0.4	130	51.9	0.4

140	53.2	0.4	150	54.2	0.5	160	55.3	0.5	170	56.2	0.5

180	57.2	0.5	190	57.9	0.5	200	58.6	0.5	210	59.2	0.5

220	59.8	0.5	230	60.4	0.5	240	61	0.5	250	61.3	0.5

260	61.7	0.6	270	61.9	0.6	280	62.1	0.6	290	62.4	0.6

300	62.8	0.6	310	63	0.6	320	62.9	0.6	330	62.9	0.6

340	63	0.6	350	62.8	0.6	360	63.2	0.6	370	63.3	0.6

380	63.2	0.6	390	63	0.6	400	62.8	0.6	410	62.7	0.6

420	62.3	0.6	430	62.5	0.6	440	62.5	0.65	450	62	0.6

460	62.8	0.7	470	62.6	0.7	480	62.3	0.7	490	62.9	0.7

500	61.8	0.7	510	61.5	0.65	520	61.4	0.65	530	61.2	0.69

540	60.9	0.7	550	60.8	0.7	560	60.5	0.7	570	60.3	0.7

580	60.6	0.7	590	59.9	0.7	600	59.9	0.7	610	60.1	0.7

620	59.5	0.7	630	58.9	0.7	640	59	0.7	650	59.6	0.7

660	58.6	0.7	670	58.9	0.7	680	58.3	0.7	690	58.6	0.7

700	58.6	0.7	710	58	0.7	720	57.6	0.7	730	57.3	0.7

740	57.1	0.7	750	57	0.7	760	57.1	0.7	770	56.5	0.7

780	56.1	0.7	790	56.1	0.7	800	55.9	0.6	810	55.7	0.6

820	55.6	0.6	830	55.4	0.6	840	55.3	0.6	850	55.1	0.6

860	55	0.6	870	54.9	0.6	880	54.7	0.6	890	54.6	0.6

900	54.3	0.6	910	54.2	0.6	920	54.1	0.6	930	54	0.6

940	53.8	0.6	950	53.7	0.6	960	53.6	0.6	970	53.4	0.6

980	53.3	0.6	990	53.2	0.6	1000	53.2	0.6	1010	52.9	0.6

1020	52.8	0.6	1030	52.6	0.6	1040	52.5	0.6	1050	52.3	0.6

1060	52.2	0.6	1070	52.1	0.6	1080	51.9	0.6	1090	51.8	0.6

1100	51.7	0.6	1110	51.5	0.6	1120	51.4	0.6	1130	51.2	0.6

1140	50.9	0.65	1150	50.9	0.65	1160	50.7	0.69	1170	50.5	0.7

1180	50.4	0.7	1190	50.3	0.7	1200	50.3	0.7	1210	50.2	0.7

1220	50.1	0.7	1230	50	0.7	1240	49.8	0.7	1250	49.7	0.7

1260	49.5	0.7	1270	49.5	0.7	1280	49.4	0.7	1290	49.3	0.7

1300	49.2	0.7	1310	49.1	0.7	1320	48.9	0.7	1330	48.8	0.7

1340	48.7	0.7	1350	48.5	0.7	1360	48.4	0.7	1370	48.3	0.7

1380	48.2	0.7	1390	48.1	0.7	1400	48	0.7	1410	47.9	0.7

1420	47.8	0.7	1430	47.6	0.7	1440	47.5	0.7	1450	47.4	0.7

1460	47.2	0.7	1470	47.2	0.7	1480	47.1	0.7	1490	47	0.7

1500	46.8	0.7	1510	46.7	0.7	1520	46.6	0.7	1530	46.5	0.7

1540	46.3	0.7	1550	46.2	0.7	1560	46.1	0.7	1570	46	0.7

1580	45.9	0.7	1590	45.8	0.7	1600	45.7	0.7	1610	45.6	0.7

1620	45.5	0.7	1630	45.4	0.7	1640	45.3	0.7	1650	45.2	0.7

1660	45.1	0.7	1670	45	0.7	1680	44.9	0.7	1690	44.9	0.7

1700	44.8	0.7	1710	44.7	0.7	1720	44.6	0.7	1730	44.5	0.7

1740	44.4	0.7	1750	44.3	0.7	1760	44.2	0.7	1770	44	0.7

1780	43.9	0.7	1790	43.8	0.7	1800	43.7	0.7	1810	43.6	0.7

1820	43.5	0.7	1830	43.4	0.7	1840	43.3	0.7	1850	43.2	0.7

1860	43.1	0.7	1870	43	0.7	1880	42.9	0.7	1890	42.8	0.7

1900	42.7	0.7	1910	42.6	0.7	1920	42.5	0.7	1930	42.4	0.7

1940	42.3	0.7	1950	42.2	0.7	1960	42.2	0.7	1970	42.1	0.7

1980	42	0.7	1990	41.9	0.7	2000	41.8	0.7	2010	41.7	0.7

2020	41.6	0.7	2030	41.5	0.7	2040	41.4	0.7	2050	41.3	0.7

2060	41.2	0.7	2070	41.1	0.7	2080	41	0.7	2090	40.9	0.7

2100	40.8	0.7	2110	40.7	0.7	2120	40.6	0.7	2130	40.5	0.7

2140	40.4	0.7	2150	40.3	0.7	2160	40.2	0.7	2170	40.2	0.7

2180	40.1	0.7	2190	40	0.7	2200	39.9	0.7	2210	39.8	0.7

2220	39.7	0.7	2230	39.6	0.7	2240	39.5	0.7	2250	39.4	0.7

2260	39.3	0.7	2270	39.2	0.7	2280	39.2	0.7	2290	39.1	0.7

2300	39	0.7	2310	38.9	0.7	2320	38.8	0.7	2330	38.7	0.7

2340	38.6	0.7	2350	38.5	0.7	2360	38.4	0.7	2370	38.3	0.7

2380	38.2	0.7	2390	38.1	0.7	2400	38.1	0.7	2410	38	0.7

2420	37.9	0.7	2430	37.9	0.7	2440	37.8	0.7	2450	37.7	0.7

2460	37.6	0.7	2470	37.6	0.7	2480	37.5	0.7	2490	37.5	0.7

2500	37.4	0.7	2510	37.3	0.7	2520	37.2	0.7	2530	37.2	0.7

2540	37.1	0.7	2550	37	0.7	2560	36.9	0.7	2570	36.8	0.7

2580	36.8	0.7	2590	36.7	0.7	2600	36.6	0.7	2610	36.5	0.7

2620	36.5	0.7	2630	36.4	0.7	2640	36.3	0.7	2650	36.2	0.7

2660	36.1	0.7	2670	36	0.7	2680	35.9	0.7	2690	35.9	0.7

2700	35.8	0.7	2710	35.7	0.7	2720	35.6	0.7	2730	35.6	0.7

2740	35.5	0.7	2750	35.5	0.7	2760	35.4	0.7	2770	35.4	0.7

2780	35.3	0.7	2790	35.3	0.7	2800	35.2	0.7	2810	35.1	0.7

2820	35.1	0.7	2830	35	0.7	2840	34.9	0.7	2850	34.9	0.7

2860	34.8	0.7	2870	34.8	0.7	2880	34.7	0.7	2890	34.6	0.7

2900	34.6	0.7	2910	34.5	0.7	2920	34.4	0.7	2930	34.4	0.7

2940	34.3	0.75	2950	34.3	0.75	2960	34.2	0.75	2970	34.1	0.75

2980	34.1	0.75	2990	34	0.75	3000	33.9	0.75	3010	33.8	0.75

3020	33.8	0.75	3030	33.7	0.75	3040	33.6	0.75	3050	33.6	0.75

3060	33.5	0.75	3070	33.5	0.75	3080	33.4	0.75	3090	33.3	0.75

3100	33.3	0.75	3110	33.2	0.75	3120	33.1	0.75	3130	33.1	0.75

3140	33	0.75	3150	32.9	0.75	3160	32.8	0.75	3170	32.8	0.75

3180	32.8	0.75	3190	33.7	0.75	3200	32.7	0.75	3210	32.6	0.75

3220	32.6	0.75	3230	32.5	0.75	3240	32.4	0.75	3250	32.4	0.75

3260	32.3	0.75	3270	32.3	0.75	3280	32.2	0.75	3290	32.2	0.75

3300	32.1	0.75	3310	32.1	0.75	3320	32	0.75	3330	31.9	0.75

3340	31.9	0.75	3350	31.8	0.75	3360	31.8	0.75	3370	31.7	0.75

3380	31.7	0.75	3390	31.6	0.75	3400	31.6	0.75	3410	31.5	0.75

3420	31.5	0.75	3430	31.4	0.75	3440	31.4	0.75	3450	31.3	0.75

3460	31.3	0.75	3470	31.2	0.75	3480	31.2	0.75	3490	31.1	0.75

3500	31	0.75	3510	31	0.75	3520	30.9	0.75	3530	30.8	0.75

3540	30.8	0.75	3550	30.7	0.75	3560	30.7	0.75	3570	30.6	0.75

3580	30.5	0.75	3590	30.5	0.75	3600	30.4	0.75	3610	30.4	0.75

3620	30.3	0.75	3630	30.3	0.75	3640	30.2	0.75	3650	30.1	0.75

3660	30.1	0.75	3670	30	0.75	3680	30	0.75	3690	30	0.75

3700	29.9	0.75	3710	29.9	0.75	3720	29.8	0.75	3730	29.8	0.75

3740	29.7	0.75	3750	29.7	0.75	3760	29.7	0.75	3770	29.6	0.75

3780	29.6	0.75	3790	29.6	0.75	3800	29.5	0.75	3810	29.5	0.75

3820	29.5	0.75	3830	29.4	0.75	3840	29.4	0.75	3850	29.4	0.75

3860	29.3	0.75	3870	29.3	0.75	3880	29.2	0.75	3890	29.2	0.75

3900	29.2	0.75	3910	29.1	0.75	3920	29.1	0.75	3930	29	0.75

3940	29	0.75	3950	29	0.75	3960	28.9	0.75	3970	28.9	0.75

3980	28.8	0.75	3990	28.8	0.75	4000	28.8	0.75	4010	28.7	0.75

4020	28.7	0.75	4030	28.6	0.75	4040	28.6	0.75	4050	28.5	0.75

4060	28.5	0.75	4070	28.5	0.75	4080	28.4	0.75	20000	5.4	0.8

Appendix D.

Audio Signal with Full Wave Rectifier, Voltages at Cell electrodes.

Hz: hz after full wave: volts at post DC

20	40	2.8:	120	240	2.5:	220	440	2.4:	320	640	2.3

420	840	2	520	1040	2.5	620	1240	2.6	720	1440	2.7

820	1640	2.7	920	1840	2.8	1020	2040	2.8	1120	2240	2.9

1220	2440	2.9	1320	2640	3	1420	2840	3.1	1520	3040	3.2

1620	3240	3.3	1720	3440	3.3	1820	3640	3.4	1920	3840	3.5

2020	4040	3.6	2120	4240	3.6	2220	4440	3.7	2320	4640	3.8

2420	4840	3.8	2520	5040	3.9	2620	5240	4	2720	5440	4.1

2820	5640	4.2	2920	5840	4.2	3020	6040	4.3	3120	6240	4.4

3220	6440	4.5	3320	6640	4.6	3420	6840	4.7	3520	7040	4.8

3620	7240	4.9	3720	7440	4.9	3820	7640	5	3920	7840	5.1

4020	8040	5.2	4120	8240	5.2	4220	8440	5.3	4320	8640	5.4

4420	8840	5.5	4520	9040	5.6	4620	9240	5.6	4720	9440	5.7

4820	9640	5.8	4920	9840	5.9	5020	10040	6	5520	11040	6.4

6020	12040	7	6520	13040	7.2	7020	14040	7.8	7520	15040	8.2

8020	16040	8.8	8520	17040	9.3	9020	18040	9.8	9520	19040	10.3

10020	20040	11	10520	21040	11.2	11020	22040	11.8	11520	23040	12.1

12020	24040	12.4	12520	25040	12.7	13020	26040	13.2	13520	27040	13.4

14020	28040	17

Appendix E.

Production and current results from an audio signal through a full wave rectifier.  With voltage held at 10 Volts.

| Hz from source | Hz after Rectifier | Volts @P | Amps @P | Ml | Delta T | Visual | Time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 40 | 10 | 0.37 | 15 | 0 | yes | 2 |  |
| 23.5 | 47 | 10 | 0.45 | 15 | 0 | yes | 2 |  |
| 26.75 | 53.5 | 10 | 0.42 | 15 | 0 | yes | 2 |  |
| 63.75 | 127.5 | 10 | 0.44 | 16 | 0 | yes | 2 |  |
| 120 | 240 | 10 | 0.37 | 15 | 0.3 | yes | 2 |  |
| 220 | 440 | 10 | 0.37 | 15 | 0.3 | yes | 2 |  |
| 320 | 640 | 10 | 0.36 | 15 | 0 | yes | 2 |  |
| 420 | 840 | 10 | 0.37 | 15 | -0.3 | yes | 2 |  |
| 520 | 1040 | 10 | 0.37 | 15 | 0.6 | yes | 2 |  |
| 620 | 1240 | 10 | 0.37 | 15 | 0.3 | yes | 2 |  |
| 720 | 1440 | 10 | 0.37 | 15 | 0 | yes | 2 |  |
| 820 | 1640 | 10 | 0.37 | 15 | 0 | yes | 2 |  |
| 920 | 1840 | 10 | 0.37 | 15 | -0.3 | yes | 2 |  |
| 1020 | 2040 | 10 | 0.37 | 15 | 0 | yes | 2 |  |
| 1120 | 2240 | 10 | 0.36 | 15 | 0.6 | yes | 2 |  |
| 1220 | 2440 | 10 | 0.36 | 15 | 0 | yes | 2 |  |
| 1320 | 2640 | 10 | 0.29 | 15 | 0.3 | yes | 2 |  |
| 1420 | 2840 | 10 | 0.29 | 15 | 0 | yes | 2 |  |
| 1520 | 3040 | 10 | 0.29 | 15 | 0.3 | yes | 2 |  |
| 1620 | 3240 | 10 | 0.29 | 15 | 0 | yes | 2 |  |
| 1720 | 3440 | 10 | 0.29 | 15 | 0 | yes | 2 |  |
| 1820 | 3640 | 10 | 0.29 | 15 | 0.3 | yes | 2 |  |
| 1920 | 3840 | 10 | 0.27 | 15 | 0 | yes | 2 |  |
| 2020 | 4040 | 10 | 0.28 | 15 | 0 | yes | 2 |  |
| 2120 | 4240 | 10 | 0.27 | 15 | 0 | yes | 2 |  |
| 2220 | 4440 | 10 | 0.27 | 15 | 0 | yes | 2 |  |
| 2320 | 4640 | 10 | 0.27 | 15 | 0 | yes | 2 |  |
| 2420 | 4840 | 10 | 0.27 | 15 | 0 | yes | 2 |  |
| 2520 | 5040 | 10 | 0.29 | 15 | 0 | yes | 2 |  |
| 2620 | 5240 | 10 | 0.29 | 15 | 0 | yes | 2 |  |
| 2720 | 5440 | 10 | 0.28 | 15 | 0 | yes | 2 |  |
| 2820 | 5640 | 10 | 0.28 | 15 | 0.2 | yes | 2 |  |
| 2920 | 5840 | 10 | 0.27 | 15 | 0 | yes | 2 |  |
| 3020 | 6040 | 10 | 0.27 | 15 | 0 | yes | 2 |  |
| 3120 | 6240 | 10 | 0.26 | 15 | 0 | yes | 2 |  |
| 3220 | 6440 | 10 | 0.17 | 15 | 0 | yes | 2 |  |
| 3320 | 6640 | 10 | 0.18 | 15 | 0 | yes | 2 |  |
| 3420 | 6840 | 10 | 0.18 | 15 | 0 | yes | 2 |  |
| 3520 | 7040 | 10 | 0.19 | 15 | 0 | yes | 2 |  |
| 3620 | 7240 | 10 | 0.18 | 15 | 0 | yes | 2 |  |
| 3720 | 7440 | 10 | 0.18 | 15 | -0.6 | yes | 2 |  |
| 3820 | 7640 | 10 | 0.18 | 15 | 0.4 | yes | 2 |  |
| 3920 | 7840 | 10 | 0.18 | 15 | 0 | yes | 2 |  |
| 4020 | 8040 | 10 | 0.18 | 15 | 0.9 | yes | 2 |  |
| 4120 | 8240 | 10 | 0.17 | 15 | 0.9 | yes | 2 |  |
| 4220 | 8440 | 10 | 0.17 | 15 | 0.2 | yes | 2 |  |
| 4320 | 8640 | 10 | 0.17 | 15 | 0 | yes | 2 |  |
| 4420 | 8840 | 10 | 0.17 | 15 | 0 | yes | 2 |  |
| 4520 | 9040 | 10 | 0.16 | 15 | -0.2 | yes | 2 |  |
| 4620 | 9240 | 10 | 0.16 | 15 | -0.3 | yes | 2 |  |
| 4720 | 9440 | 10 | 0.15 | 15 | -0.3 | yes | 2 |  |
| 4820 | 9640 | 10 | 0.15 | 15 | 0 | yes | 2 |  |
| 4920 | 9840 | 10 | 0.15 | 15 | 0 | yes | 2 |  |
| 5020 | 10040 | 10 | 0.14 | 16 | 0 | yes | 2 |  |
| 9840 | 19680 | 10 | 0 | 16 | 0 | yes | 2 |  |

Appendix F.

Production and current results from an audio signal through a full wave rectifier.

| Hz from source | Hz after Rect | Volts @P | Amps ma @P | Ml | Delta T | Visual | Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 40 | 10 | 32 | 10 | 0 | yes | 1 |
| 23.5 | 47 | 10.8 | 40 | 14 | 0 | yes | 1 |
| 26.5 | 53 | 13.7 | 60 | 14 | 0 | yes | 1 |
| 63.75 | 127.5 | 13.5 | 60 | 15 | 0 | yes | 1 |
| 120 | 240 | 16.8 | 54 | 22 | 0 | yes | 1 |
| 220 | 440 | 18.4 | 50 | 15 | 2.7 | yes | 1 |
| 320 | 640 | 18.9 | 40 | 30 | 0.6 | yes | 1 |
| 420 | 840 | 19.1 | 36 | 30 | 1.2 | yes | 1 |
| 520 | 1040 | 19.3 | 32 | 31 | 0.3 | yes | 1 |
| 620 | 1240 | 19.4 | 25 | 32 | 0.5 | yes | 1 |
| 720 | 1440 | 19.2 | 24 | 32 | 0.6 | yes | 1 |
| 820 | 1640 | 19 | 22 | 33 | 0.9 | yes | 1 |
| 920 | 1840 | 18.9 | 21 | 32 | 1.2 | yes | 1 |
| 1020 | 2040 | 18.8 | 20 | 35 | 0.9 | yes | 1 |
| 1120 | 2240 | 18.8 | 19 | 36 | 0.3 | yes | 1 |
| 1220 | 2440 | 22 | 19 | 36 | 0 | yes | 1 |
| 1320 | 2640 | 21.3 | 19 | 38 | 1 | yes | 1 |
| 1420 | 2840 | 20.7 | 18 | 38 | 1.8 | yes | 1 |
| 1520 | 3040 | 20.7 | 18 | 38 | 1.2 | yes | 1 |
| 1620 | 3240 | 20.5 | 14 | 35 | 1.5 | yes | 1 |
| 1720 | 3440 | 20.5 | 13 | 34 | 1.2 | yes | 1 |
| 1820 | 3640 | 20.3 | 13 | 32 | 0.9 | yes | 1 |
| 1920 | 3840 | 20.6 | 13 | 38 | 1.8 | yes | 1 |
| 2020 | 4040 | 20.5 | 13 | 35 | 1.2 | yes | 1 |
| 2120 | 4240 | 20.2 | 11 | 35 | 0.9 | yes | 1 |
| 2220 | 4440 | 20 | 11 | 35 | 1.2 | yes | 1 |
| 2320 | 4640 | 19.8 | 11 | 34 | 1.5 | yes | 1 |
| 2420 | 4840 | 19.6 | 11 | 30 | 0.8 | yes | 1 |
| 2520 | 5040 | 19.4 | 10 | 31 | 0.9 | yes | 1 |
| 2620 | 5240 | 18.1 | 10 | 30 | 0.6 | yes | 1 |
| 2720 | 5440 | 18.1 | 10 | 30 | 0 | yes | 1 |
| 2820 | 5640 | 18.1 | 10 | 30 | 0 | yes | 1 |
| 2920 | 5840 | 18 | 10 | 29 | 0 | yes | 1 |
| 3020 | 6040 | 17.9 | 10 | 29 | 0 | yes | 1 |
| 3120 | 6240 | 17.7 | 10 | 29 | 0.6 | yes | 1 |
| 3220 | 6440 | 17.6 | 10 | 28 | 0.6 | yes | 1 |
| 3320 | 6640 | 19.9 | 10 | 29 | 0.6 | yes | 1 |
| 3420 | 6840 | 20.1 | 10 | 29 | 0.7 | yes | 1 |
| 3520 | 7040 | 19.8 | 10 | 32 | 0.9 | yes | 1 |
| 3620 | 7240 | 19.6 | 10 | 29 | 1.2 | yes | 1 |
| 3720 | 7440 | 19.1 | 9 | 29 | 1.5 | yes | 1 |
| 3820 | 7640 | 19.1 | 9 | 29 | 0.6 | yes | 1 |
| 3920 | 7840 | 18.9 | 9 | 29 | -0.1 | yes | 1 |
| 4020 | 8040 | 18.8 | 9 | 30 | 0 | yes | 1 |
| 4120 | 8240 | 18.9 | 9 | 29 | -0.3 | yes | 1 |
| 4220 | 8440 | 18.7 | 6 | 30 | -0.1 | yes | 1 |
| 4320 | 8640 | 18.5 | 6 | 29 | 0 | yes | 1 |
| 4420 | 8840 | 18.3 | 6 | 28 | 1.5 | yes | 1 |
| 4520 | 9040 | 18.3 | 6 | 29 | 1.5 | yes | 1 |
| 4620 | 9240 | 18 | 6 | 29 | 1.8 | yes | 1 |
| 4720 | 9440 | 17.8 | 5 | 29 | 0.9 | yes | 1 |
| 4820 | 9640 | 17.7 | 5 | 28 | 1.6 | yes | 1 |
| 4920 | 9840 | 17.6 | 5 | 28 | 0.6 | yes | 1 |
| 5020 | 10040 | 17.2 | 6 | 29 | 0.9 | yes | 1 |
| 11220 | 22440 | 17.2 | 0 | 28 | 1.1 | yes | 1 |
| 11320 | 22640 | 16.4 | 0 | 21 | 0 | yes | 1 |
| 16220 | 32440 | 16.7 | 0 | 25 | 0 | yes | 1 |

Appendix H.

Power and Current info from Morse (2016).

The use of an audio amp to provide an A.C. signal at the resonant frequency of the cell. The resonance frequency is found by recording the metal of the cell after being struck. Then the file is analyzed with an audio frequency analyzer to determine resonant frequencies.  The resonance frequencies are found to be 46.7. 53, and 127 Hz.  A signal from a frequency generator was running through an audio amplifier, in this way an alternating current at the desired frequency was obtained. These tests show that as the frequency current draw dropped to zero while still producing HHO gas.

Appendix I. (Morse, 2016)

Audio Signal through a full wave rectifier.

A full wave rectifier if a diode bridge that in essence splits the positive and negative portions of the alternating wave. It then inverts the negative portion to positive. The end result is a positively charged pulse at twice the frequency rate.

It is shown that the voltage value of the rectified audio signal follows the characteristics of the continuous d.c. voltages as shown in the Volts at posts in the DC graph to the right.  As a result, it is possible to find a frequency that will have a very small current draw using an audio signal rather than a PWM which may require advanced electronics.

Results: Production and current results of using an audio signal through a full wave rectifier with the voltage held at 10 volts. Appendix (G).

Appendix J.

Discussion on Experiments

It is shown that voltage seems to be the deciding factor on production. No matter what Hz the production value was stable at 7.5 mL per minute.  An unexpected outcome is a drastic reduction in current needed. Current dropped to 50 milliamps or less, also it was noticed that at extremely high frequencies production remains the same but amperage draw drops to zero. This is very promising and greatly reduces the cost of producing HHO.  However, 7.5 mL per minute is not enough production to supply the need.  Production and current results of using an audio signal through a full wave rectifier with only Hz being changed and voltage and amperage being the result of the given Hz. Appendix (H) (Morse, 2016). Again the results show that at 22440 Hz current draw drops to zero.  While the production did reduce by 2 mL per minute this is an acceptable loss to have the net cost drop so radically. Delta T (temperature increase) is also lower at this point.  The other alternative is to use a Pulse Width Modulator, and from our research, the current reduction can also be achieved.

Appendix K.

New York State Data

https://data.ny.gov/Energy-Environment/Greenhouse-Gas-Emissions-From-Fuel-Combustion-Mill/djfn-trk4

Year		Residential Total	Commercial Total	Industrial

Total	Transportation Total	Electric Generation Total	Net Imports of Electricity	Year Total

2012		30.7	21	10.6	68	32.2	9.5	171.9

2011		31.4	24.3	14.9	72.2	33.4	9.1	185.3

2010		32.2	25.1	10.5	73.1	37.3	9.6	187.8

2009		33.4	25.4	11.5	76.7	34	9	190.1

2008		35.1	25.8	13.8	80	42.5	8.6	205.9

2007		36.9	26.7	14.1	81.8	48.8	7.5	215.8

2006		33	25.4	14.6	84.1	45.9	6.7	209.9

2005		39.7	28.6	15	84	53.6	6.6	227.5

2004		38.8	34.8	14.1	82.9	52	6.4	228.9

2003		39.3	32.8	14	82.7	51.7	6.6	227.2

2002 		36.7	31.1	14.9	80.6	50.7	6.2	220

2001		38.9	30.7	16.3	78.3	54.9	3.8	223

2000		40.2	32.2	17.5	78.9	55.7	5.7	230.1

1999		35.1	30.3	18	77.3	56.4	2.5	219.7

1998		32.4	27.8	21.7	75	56	1.1	214

1997		35.6	29.8	22.5	72.8	52.3	1.8	214.8

1996		37.2	27.9	23.3	72.5	46.8	3.4	211.2

1995		34.9	27	22.5	67.7	51.3	4.3	207.6

1994		35.9	27.8	20.6	65.5	47.4	6.7	203.9

1993		36.4	28.3	20.4	65.3	47.6	7.1	205

1992		36.8	27.4	20.7	63.4	53.8	6	208.2

1991		33.4	26.3	19.4	62.3	58.7	3.4	203.6

1990		34.2	26.5	20	62	63	1.6	207.3

New York State emissions Data

Data for Buffalo N.Y. https://data.ny.gov/Energy-Environment/Title-V-Emissions-Inventory-Beginning-2010/4ry5-tfin

2013	Erie	Buffalo	9140200021	AURUBIS BUFFALO INC

SIC Code  VOC (tons)  NOx (tons)  CO (tons)  CO2 (tons)  Particulates (tons)  PM10 (tons)	PM2.5 (tons)

3351	    15.46 	12.43	   978.18       35779.68	             8.45	            3.76	       2.97

HAPS (tons)	SO2 (tons)

0		0.18	"	Buffalo, NY       (42.887691, -78.879374)"

Appendix L.

Background - Various forms of energy.

“Energy is a quantity that can be assigned to every particle, object and system of objects as a consequence of the state of that particle, object or system (Harper, 2007).” Kinetic, potential, thermal, gravitational, sound, elastic, light and electromagnetic energies are all different forms of energy. The production of energy is important in all areas of the economy. To add to this, per capita, energy consumption is directly related to our standard of living (Vader and Joshi, 2005).  Currently being researched by science are alternative energy sources such as solar, wind, biogas/biomass, tidal, geothermal, fuel cell, hydrogen energy, small hydropower, etc. (Alias, 2005).  This paper will focus on Hydrogen as (HHO) and Electric as a way of reducing emissions while allowing the four horseman to retain power by not moving entirely from fossil fuels. Biofuels, however, can be made from vegetable oils, animal fats or algae and is commonly referred to as Biodiesel, an alternative diesel fuel. These are renewable biological sources that are normally biodegradable and non-toxic. Biomass is a bio-source of H2 and water that is burnable through a gasifier. This is the best alternative in my opinion as humans generally pay to get rid of all our biomass so why not make fuel from it. Electricity is both a power that can be produced cleanly and used cleanly.  All these can have low emission profiles and be environmentally beneficial (Krawczyk, 1996).  However, due to these characteristics, researchers are focusing much attention on hydrogen as an alternative fuel in ICEs and biomass is a great source of hydrogen (Saravanan and Nagarajan, 2008).

Background - Hydrogen energy.

Though H2 has been looked at as an alternative because it enhances engine efficiency and produces less pollution (Boretti, 2010), the expenses to include this in manufacturing is too high (White et al, 2006).  H2 can be produced from electrolysis of water, coal gasification, from biomass, and solar photoelectrolysis (Saravanan and Nagarajan, 2008).  However, though H2 is one of the most common elements in the universe it is rarely found in nature by itself.  Using H2 can theoretically extend the lean limit—the least amount necessary to combust-- of a fuel mixture.  This is done simply by adding a small amount of hydrogen to a liquid or gaseous fossil fuel. In turn, a complete combustion occurs which improves efficiency and decreases NOx (Jingding et al., 1998)(Stebar and Parks, 1974).  To add to this, during ultra-lean operation NOx formation rates are so low that engine out emissions is near zero (Das, 1991).  On the other hand, there are several problems with using H2 alone as a fuel, foremost of which is the cost of production as we mentioned but there is another way to burn H2.

Background - Brown’s Gas

Brown’s Gas is a mixed form of H2 and O2 that is easily produced at a fraction of the cost of producing pure H2. Brown’s Gas has a few aliases: Hydroxy, HHO, and H2O2 to name a few.  Basically, it is the vapor from the water after electrolysis. Scientifically, Hydroxy Gas (Brown's Gas) is a mixture of monoatomic and diatomic hydrogen and oxygen referred to as “Electrically Expanded Water (EEW)” or “Santilli Magnecules”.  Brown’s Gas has a cool flame of about 130°C yet is able to melt just about anything. Although the flame is cool Brown’s Gas can fuse brick, steel, sublimated tungsten, glaze quarts, and neutralize nuclear waste and burns with a clean flame. It uses no atmospheric oxygen and creates only pure water as its combustion product. (Michrowski, 1993).  This gas cannot be stored safely it is very volatile and highly explosive at standard temperatures and pressures when mixed with air (Cameron, 2012). As a result, the forming and implementation must be done without ever storing or pressurizing the gas, i.e., it must be used as it is produced. This necessitates the need for a production unit or--HHO cell--for every application.  When water is electrolyzed (when an electric current is passed through) Rydberg Clusters may be formed. Clusters of hydrogen and oxygen including water molecules in the “highly energized trigonal-by pyramidal geometry, monatomic and diatomic hydrogen, free electrons and oxygen” (Eckman, 2010).  Rydberg clusters are in solids and liquids and are very stable for hours. In the case of HHO or Brown’s Gas, these clusters have shown a lifespan of about 10 hours (Santilli, 2006). These clusters make use of the hydrogen bond which is a relatively weak bond when compared to the covalent bond (Mccarthy, 2008).

Background - HHO Cells

There are two types of HHO cells currently. The Dry Cell, where only the center of the plates have an electrolyte, and the Wet Cell, where the entirety of the plates are submerged. The advantages of the dry cell are; the water is less, generated heat is smaller due to the circulation between cell and reservoir and easy access for testing plate voltages. The electric current used is also smaller because less of the power is converted into heat. The plate usage is only 60% versus 100% in a wet cell.

The Wet Cell is a generator with the electrodes fully immersed in the electrolyte. The advantages of the wet cell are;  gas production has more quantity, the flow is stable, construction and maintenance are easier (Bambagn et al, 2016).

Background - Stakeholders

Since HHO production is not centralized, and storage is dangerous, it cannot be metered and monopolized. As a result, such technology will be in offense to the four horseman stakeholders because they cannot sell it. To add the increase in fuel economy will directly cut into their profits.  It is Tesla’s dilemma all over again. Since the population at large is our other stakeholder let us hope that we have better luck than him. Remember Tesla died bankrupt and alone in a small apartment after all his patents were stolen and used for profit, although, his science is still changing the world today, after all who does not have a cell phone?  My only solution to this problem is that the changes be small.  Unable to address the first foreseen problem more than that, we will concentrate on the other foreseen problems.  Stockholm environment institute suggests we might begin by simply limiting the amount of permitted fossil fuel licenses we allow. In their declaration that avoiding dangerous climate change requires rapid transition away from fossil fuels and that a full phase-out of global fossil fuel consumption – particularly coal and oil—will have to be completed within the next 50 years (Rogelj et al, 2015).  This report shows a tentative reduction in CO2 emission by simply limiting the permitted extractions of fossil fuels. Under such a policy coal production in the U.S. would steadily decline. To add to this oil and gas extraction would drop as well. They conclude by stating that the phasing out of federal leases for fossil fuel extraction would reduce global CO2 emissions by 100 million tons per year. These findings suggest that policy-makers should give greater attention to slowing down the permitting of expansion of fossil fuel extractions. However, fossil fuel expansion is at an all-time high and are only moving upwards (Rystad, 2015).  Our current scheme of investing in fossil fuel expansion will lock us in the long-term making it impossible to recover from the effects of the effects of a fossil fuel based economy (Erickson et al. 2015).

Background - Internal Combustion Fundamentals.

The engine used mostly powered by burning fossil fuel is the internal combustion Engine (ICE). These come in two main models the spark ignition (SI) known as a gasoline engine, and the compression-ignition (CI) engine known as the diesel engine. Each of these come in a range of styles and types. The four stroke, the two-stroke, and rotary are among these types; the stroke is defined as how many times the piston travels up and down in one cycle. We will be dealing with the four stroke models mostly to help us understand engine fundamentals (Kahraman, 2005; Haywood, 1998). The following pictures are from Haywood, 1998.

The picture on the left references the four stroke per cycle engine, and the picture on the right has two typical types for fuel carburetion. Basically, fuel travels through the carburetor or injector to the cylinder on the intake down stroke. The fuel is compressed on the following compression upstroke. Combustion occurs either due to spark on SI engines of compression on CI engines which pushes the piston down on the combustion stroke and the leftovers are evacuated on the last upstroke called the exhaust stroke. Then the process repeats (Haywood, 1998). In our literature reviews, engine torque will normally be measured with a dynamometer. The engines output shaft is coupled to the dynamometer much like a transmission is to an auto. Then torque coefficient is found using an equation.

P (kW) = 2p w (rev / s) ´ T (Nm) x 10-3 brake mean effective pressure (bmep ).

where,

When doing the math it is found that a perfect energy conversion is not attainable. The best that is found is about 96% (Kahraman, 2005; Haywood, 1998) (Yilmaz, 2010).

Background - Electrons, Water, and Conductivity

Water H2O is a compound that is most important in life.  Because of this, I am against using water hydrolysis as a formal means of producing power.  I much prefer reclamation and the recycling of Biomass and human and animal waste as the formal means of producing H2, HHO, Methane, or other usable sources of fuel. However, that being said, let the research go on. Water consists of a compound of 2 molecules hydrogen (H2)and one molecule Oxygen. When separated we have 2H2O -> 2H2+ O2. Since this is so simple and lite, compared to the much heavier petroleum, we have a much greater bang for the buck.  Water can be turned into HHO gas through electrolysis (Durairai et al, 2012).  Hydrogen has a calorific value of 120MJ/kg, a value that is much greater than gasoline, diesel or natural gas (Verhelst, 2009).  Electrolysis is achieved by passing an electric current through two electrodes submerged in water.  Electrolysis occurs when DC electron flows from a cathode to the anode via an electrolytic solution (Bureau, 2011). However, this produces heat so to reduce heat often an electrolyte is used as a catalyst. A bubbler unit must be used to prevent flashback.  Hydrogen at or above 2500psi can easily lead to a blast.  Using DC current the electrodes have two poles; a positive--the anode, and negative-- the cathode.  The following reactions are found:

Base equilibrium reaction:

Cathode (Reduction) 2H2O(l)+2e-H2(g)+2OH-(aq)

Anode (Oxidation) 4OH-(aq)O2(g)+2H2O(l)+4e-

Overall reaction 2H2O(l)2H2(g)+O2(g)

Acid equilibrium reaction:

Cathode (reduction) 2H+ (aq) + 2e-  H2 (aq)

Anode (Oxidation) 2H2O (l)  O2 (g) +4H+ (aq) +4e-

(Bambagn et al, 2016)

Background - Bio Diesel

Biodiesel is diesel fuel made from vegetable oils, animal fats, recycled restaurant greases, algae, and other biomass. It is normally safe, biodegradable, and produces fewer air pollutants than petroleum-based diesel. Biodiesel can be used in its pure form or blended with petroleum diesel.  Bio Diesel production Transesterification can be performed by continuous or batch systems.  It is a process where vegetable oil or other biomass is reacted with alcohol or methanol and is catalyzed by bases, acids or enzymes to form esters and glycerol. The viscosity of the oil is changed to something that is very close to petroleum diesel (Durairai et al, 2012).  Drop in replacement for diesel or CI engine of biodiesel with HHO is thought to be a valid alternative for the workhorses of our nation.

Bambagn et al manufactured a PWM to control temperature. Instead of water injection to cool the cylinder, it was thought that Pulse width modulation temperatures might be controlled better. A PWM is an electronic circuit that is able to regulate current input by quickly turning the current on and off. This duty cycle and frequency can be set and adjusted (Ghiffari, 2013).  One goal is to keep the Cell below 60° C (Musmar et al, 2011). The objective of this study is to see if these advantages remained with original engine specifications. A Dry Cell HHO device was used and optimization of PWM was done by varying the duty cycle of pulse width modulation i.e. 20%, 40%, 80%, and 100%. A venturi style mechanism mounted intake was used to port HHO. Measured was the effects of HHO gas on the performance and the temperature of the Cell. The results show optimum performance is generated by a PWM system with a 40% duty cycle. Documented increase BMEP, thermal efficiency, engine torque, power. The thermal efficiency increased respectively to 2.27%, 2.76%, and 3.05% while the performance increased by 6.55%, 7.65%, and 15.50%.  The electrolyte used is an alkaline solution of KOH. The HHO Gas generator is composed of two basic components. Tube generator and a power source (battery). A Dry cell type generator with a separated bubbler and water fill (Bambagn et al, 2016).

The Performance parameters (taken directly from the study) (Bambagn et al, 2016).

- Generator Power Input.  The Formulation to find the input power is P=VxI. Watts=volts x amps.

- HHO Gas production= measured by a gas flow meter.

- Specific Energy Input defined as the amount of energy required to process the electrolysis of water in kjoule to produce 1kg of HHO gas.

- Generator efficiency, the ratio of useful energy to the energy supplied to the system.

- Generator HHO Temperature

- The PWM system. PWM is an electronic circuit to control the amount of electric current that enters the equipment and to avoid excessive power dissipation in the battery. Also, Voltage is regulated by percentage of pulse width to the period of a square signal in the form of a periodic voltage applied to the motor as a power source. PWM signal can be constructed using analog methods using op-amp circuit or by using a digital method that could be affected by the resolution of the PWM itself. PWM electronic circuits can be make using a 555 timer ic or IC LM324N.  Timer IC is one type of timer that has the ability PWM controller with pulse width control features 0 to 100%.  Mosfet drivers are needed to the use of PWM.

Results

Generator temp was maintained at 60 ° C.  Application of HHO gas generator in point above on standard ignition timing engine produce in an increase of performance such as torque, power, BMEP and thermal efficiency respectively of 2.27%, 2.76%, and 3.05% and decrease of bsfc 7.76% (Bambagn et al, 2016).

Ammar Template and Design,

The type cells used in this experiment was a Wet Cell filled with water and sodium bicarbonate. Two cells were constructed being the same type with different configurations FCb and FCc.  Cell FCc is half the dimension of cell b, with an electrode placed closer.

Results, (list is taken directly from study)

- An increase in efficiency of about 3% for cell b and 8% for cell c.

- Fuel consumption was reduced especially with cell c.

- fig 10 shows a reduction in exhaust gasses this leads to better combustion and cleaner gas

- Increased power.

Conclusions

- The use of HHO in gasoline engines enhances combustion efficiency, consequently reducing fuel consumption and thereby decreasing pollution.

- The optimal size of the FC is when the service area of an electrolyte needed to generate sufficient amount of HHO is twenty times that of the piston surface area. Also, the volume of water needed in the cell is about one and a half that of the engine capacity.

The FC which can be used in simple, easily constructed, and easily integrated with existing engines at low costs, about 15 dollars per cylinder. How does design affect production?

(Ammar, 2010)

Appendix M.

Sites searched.

http://www1.chapman.edu/library/indexes/othercatalogs.html

http://www.nspe.org/

http://www.worldcat.org/

http://www.teachthought.com/learning/100-search-engines-for-academic-research/

http://www.techrepublic.com/blog/10-things/10-tips-for-smarter-more-efficient-internet-searching/

https://scholar.google.com/intl/en-US/scholar/help.html

http://www.tubechop.com/watch/3146370

http://library.uoregon.edu/guides/history/primary.html

https://scholar.google.com/

http://deeperweb.com/

http://deep-web.org/how-to-research/deep-web-search-engines/

http://www.wikihow.com/Search-the-Deep-Web

http://techdeepweb.com/4.html

http://www.educatorstechnology.com/2013/02/12-fabulous-academic-search-engines.html

https://academic.microsoft.com/

http://www.base-search.net/about/en/

http://citeseerx.ist.psu.edu/index;jsessionid=A45573E5074EF5766CC6C13B1E3FAB86

https://doaj.org/

http://journals.plos.org/plosone/

http://www.bioone.org/

http://iopscience.iop.org/1468-6996/

http://iopscience.iop.org/1367-2630

http://www.sciencedirect.com/

http://www1.chapman.edu/library/

https://jobs.ca.gov/Public/VacancyBy.aspx?q=JNjhqjoqIhuZ8k%2fyJxpO4SNBhwX%2fKeE3n2VxdAp16%2bNPO6HtC3dCZlQdNzq%2bXgtue3tfxeEn5dds8wb7uDzS7Pbzff1dHXOrv6VhwTKPphmZs75bRCMeHOpNQTX3GQbC87F6tbltgU4H7Ldts3k5hSE7NQyYgxfdWf7B67%2fm1%2fs%3d

https://www.academia.edu/